;Programma per il lampeggio di un LED, la temporizzazione di lampeggio

;� effettuata con il Timer0 interno

PROCESSOR	16F84A

INCLUDE		"P16F84A.INC"

__CONFIG	3FF1H

RADIX		DEC

ORG		20H		;Inizio area variabili

COUNT		RES		1		;Generic Counter

COUNTH		RES		1		;Timer Counter

#DEFINE		LED		PORTA,2		;Output

#DEFINE		Page_1		STATUS,RP0	;Page 1 bank switch

ORG		0		;Start of code

GOTO		INIT

;DELAY; Per calcolare la costante diritardo: Ritardo voluto= 20 millisecondi

;20000 microsecondi con il prescaler a 1:256 diventano 20000/256=78

;conteggi del timer che deve essere presettato in modo che vada in overflow

;dopo 78 cicli, quindi deve essere programmato a 256-78=178 che � il

;valore da cui il timer deve cominciare a contare per avere 78 cicli fino

;ad arrivare ai 256 cicli permessi

DELAY		MOVWF		COUNT

DEL01		MOVLW		.178

MOVWF		TMR0

BCF		INTCON,T0IF

DEL02		BTFSS		INTCON,T0IF

GOTO		DEL02

DECFSZ		COUNT,F

GOTO		DEL01

RETURN

;Program Init

INIT		MOVLW		0H		;xx00 0000 Data Direction

BSF		Page_1		;Set Page 1

MOVWF		TRISA		;I/O Port

MOVLW		47H		;0000 0111

MOVWF		OPTION_REG	;Presc TMR0/256

BCF		Page_1		;Set Page 0

CLRF		PORTA		;Clear output

;Main program start

START		BSF		LED		;Accende LED

MOVLW		.20		;20*20 msecondi=0.4 secondi

CALL		DELAY		;Tempo LED spento

BCF		LED		;Spegne LED

MOVLW		.20		;40*20 msecondi=0.8 secondi

CALL		DELAY		;Tempo LED spento

GOTO		START

END