Speed Controller Programming Instructions

For Advance programmable (Heli version)

H70A/H100A Parameters:

| ESC for Helicoptor | Cons -Current | Max Current | BEC | Input | TIMING | PWM | Governor mode | Cut-off Voltage | Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H70A | 70A | 80A | 5V 5A UBEC | 6-18NimH ~2-6 LiPo | Programmable – 1/7/15/30 | Programmable – 8/16K | Programmable | Nimh: 60% of staring vol; Lipo: 2.8V per cell | 75g |
| H100A | 100A | 125A | 5V 5A UBEC | 6-18NimH ~2-6 LiPo | Programmable – 1/7/15/30 | Programmable – 8/16K | programmable |  | 120g |
| Other features: *Soft star * Governor mode programmable (4 options: Disable, 2-4 poles, 6-10 poles, 12-14 poles motor) * ESC slow down at 3V per cell lipo and Cut –off voltage at 2.8V per cell |  |  |  |  |  |  |  |  |  |

·Phrases 1 Enter programming Mode

    - Connect your motor and receiver to the controller. Don’t connect the battery yet.

    - Turn on your transmitter and move the throttle stick to the full throttle position (full up). Please Note: Most Futaba transmitters have the throttle channel reversed by default.

    - Connect your battery then the controller will initialize with a musical tone.

·Phrases 2 Programming

After 3 seconds, the controller will start beeping a sequence of tones – a musical tone followed by one or more beeps. Each sequence represents a parameter that you can program and is repeated 3 times. The parameters are:

| ♪— | Music Tone + 1 Beep | Options 1. Cell Type and No. of Cells |
| --- | --- | --- |
| ♪— — | Music Tone + 2 Beeps | Options 2. Throttle Setting |
| ♪— — — | Music Tone + 3 Beeps | Options 3. Throttle type |
| ♪— — — — | Music Tone + 4 Beeps | Options 4. Direction and Cutoff Type |
| ♪— — — — — | Music Tone + 5 Beeps | Option 5. Timing Mode |
| ♪— — — — — － | Music Tone + 6 Beeps | Option 6. PWM setting |

Step 1. Starting, Enter Sub-optins. When you hear the sequence for the parameter you wish to program, move the throttle stick to the Center Position to Enter Sub-options.

The controller will then start beeping a Morse code sequence of short and long beeps representing the possible options you may choose for the selected parameter. See table 2 for a list of all programmable options. Each option sequence is repeated 3 times.

Step 2. Select and save, the select the option, move the throttle stick back to the Full-up-position., When you hear the sequence for the option you wish to select. The controller will then save the selected option, and sound a long beep as a confirmation. It then goes back to the beginning of the programming sequence (phrases 2).

Step 3. Complete programming and save options. Setup all the parameters you need to change. When complete, move the throttle stick to the Lowest (Down) Position. The controller will save all options and re-initialize in normal running mode so you can start your motor.

The table below summarizes the various programming options for each parameter:

| Option 1 (2S-6S)-ESC Cell Type and Number of Cells ♪— | H70A/H100A (2-6s Lipo) |
| --- | --- |
| • — 1 Short + 1 Long | NiMh/NiCD Auto Cell Count - 0.8V/Cell Cutoff Voltage |
| • — — 1 Short + 2 Long | 6S Li-Po (22.2V) –16.8V Cutoff Voltage |
| • — — — 1 Short + 3 Long | 5S Li-Po (18.5V) – 14V Cutoff Voltage |
| • — — — — 1 Short + 4 Long | 4S Li-Po (14.8V) – 11.2V Cutoff Voltage |
| • — — — — —1 Short + 5 Long | 3S Li-Po (11.1V) – 8.4V Cutoff Voltage |
| • — — — — — — 1 Short + 6 Long | 2S Li-Po (7.4V) – 5.6V Cutoff Voltage |

| Option 2. Throttle Setting ♪— — |  |
| --- | --- |
| •• — 2 Short + 1 Long | Auto Throttle Range * |
| •• — — 2 Short + 2 Long | 1.1ms to 1.8ms |
| •• — — — 2 Short + 3 Long | Hard Acc* |
| •• — — — — 2 Short + 4 Long | Soft Acc |

| Option 3. Throttle Type ♪— — — |  |
| --- | --- |
| ••• — 3 Short + 1 Long | Normal （Disable Governor Mode）* |
| ••• — — 3 Short + 2 Long | Governor Mode with 2-4 poles motors |
| ••• — — — 3 Short + 3 Long | Governor Mode with 6-10 poles motors |
| ••• — — — — 3 Short + 4 Long | Governor Mode with 12-14 poles motors |

| Option 4. Direction and Cutoff Type ♪— — — — |  |
| --- | --- |
| •••• — 4 Short + 1 Long | Clockwise Rotation * |
| •••• — — 4 Short + 2 Long | Counterclockwise Rotation |
| •••• — — — 4 Short + 3 Long | Soft Cutoff |
| •••• — — — — 4 Short + 4 Long | Hard Cutoff * |

| Option 5. Timing Mode Setting ♪— — — — — |  |
| --- | --- |
| ••••• — 5 Short + 1 Long | 1º - For 2-4 Pole Inrunner Motors * |
| ••••• — — 5 Short + 2 Long | 7º - For 6-8 Pole Motors |
| ••••• — — — 5 Short + 3 Long | 15º- For 10-14 Pole Outrunner Motors |
| ••••• — — — — 5 Short + 4 Long | 30º - For 10-14 Pole High-RPM Outrunner Motors |

| Option 6. Pulse Width Modulation(PWM) Setting ♪— — — — — — |  |
| --- | --- |
| •••••• — 6 Short + 1 Long 8KHz | – For low RPM and low pole count motors * |
| •••••• — — 6 Short + 2 Long 16KHz | – For most out runner motors |

* is Default Setting