| RS-550PC/VC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Carbon-brush motors |  |  |  |  |  |
| OUTPUT:5.0W-100W (APPROX) |  |  | WEIGHT: 247g (APPROX) |  |  |  |
| Typical Applications : |  | Cordless Power Tools> Drill / Cordless Garden Tool / Air Compressor Toys and Models> Ride-on Toy |  |  |  |  |

*By clicking the "MODEL", you can display the Performance Chart Simulation.

| MODEL |  | VOLTAGE |  | NO LOAD |  | AT MAXIMUM EFFICIENCY |  |  |  |  | STALL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | OPERATING RANGE | NOMINAL | SPEED | CURRENT | SPEED | CURRENT | TORQUE |  | OUTPUT | TORQUE |  | CURRENT |
|  |  |  | V | r/min | A | r/min | A | mN·m | g·cm | W | mN·m | g·cm | A |
| RS-550PC | 8019(*1) | 6.0 - 9.6 | 7.2 | 15300 | 1.40 | 13540 | 10.8 | 41.7 | 425 | 59.1 | 363 | 3700 | 83.0 |
| RS-550VC | 7525(*1) | 6.0 - 14.4 | 12 | 17600 | 1.20 | 15730 | 10.1 | 58.3 | 594 | 95.9 | 549 | 5596 | 85.0 |

(*1) CCW shifted commutation (CCW+)

UNIT: MILLIMETERS

| RS-550PC-8019 | 7.2V |
| --- | --- |

| RS-550VC-7525 | 12.0V |
| --- | --- |